Rubber dam isolation

Rubber dam provides isolation for complete tooth , isolation is important before RCT.

*Dr. Jamal prefers to do the access cavity before placing the rubber dam , in order to know the long access of the tooth , and whether it’s mesially or distally tilted . So we do the access cavity then put the rubber dam immediately, don’t put any instrument in the canal before placing the rubber dam , the only instrument can be used before is the explorer / probe.

Rubber dam should cover the whole tooth  , and  leave a space for the patient to breathe , otherwise the patient may faint.

Reasons for using the rubber dam :

- Protection

-from  swallowing any instrument that might split and go in the patients mouth.

-from injury caused by rotary and hand instruments.

-from swallowing the irrigant which is the sodium hypochlorite “not the normal saline that we use in the clinics” which has a very bad taste . So we are preventing the  sodium hypochlorite from going into the patients mouth .

- efficiency

-improve visibility, since everything is covered except the tooth.

- prevent salivary leakage thus provide dry field.

- prevent fogging of the mirror when we are working on the upper jaw.

-relax the patient who fears from swallowing instruments or from suffering unpleasant taste of  the irrigant.

-minimize, actually prevent conversations and the need for frequent rinsing.

- cross infection risk

Esp. for AIDS , hepatitis and TB patients we need a good isolation .

- Legal considerations.

Equipments of rubber dam  :

- Rubber itself

It comes with different colors and thicknesses , We have the light, medium, heavy, and extra heavy. We don’t use the light one , because it tears easily nor the heavy one , because it needs force to be placed onto the frame. So we prefer the medium thickness one . regarding it’s size , we use the 6*6 inches because it provides good cover , 5*5 inches is used for pediatrics.

2-frame

We have metal and plastic frames, when they started using

rubber dam they started with metal frames but these were

appear in the radiograph  and cover the structures behind ,so  plastic ones is used to take the x-ray while the rubber dam is present.

3-clamp / retainer

We have winged “which we use in the clinics” and  other wingless clamps.

No.9 is the butterfly shape which is used for anterior teeth

No. 2/ 2A is used for premolars.

No.14/14A  is used for molars.

Numbers vary from one company to another and the previous numbers are according to ivory company.

We have also what is called deep reaching clamps that have apically inclined peaks and these are used when little coronal tooth structure is remaining to catch deeper below the gum line , but not below the crestal bone because if it reaches there then the tooth needs extraction instead of  RCT .

- Rubber dam punch

The instrument that make hole in the rubber, we use the biggest hole.

- Forceps

Used to hold the clamp in order to place the rubber dam on the tooth structure.

Application of the rubber dam :

- Put the rubber on the frame.

- Make a hole in the middle regardless the tooth you are working on.

- Put the clamp on the rubber by inserting the wings in the hole.

- Open the clamp by the forceps applying force on it to catch the under cut.

- Release the wings from the rubber that is on , to go down the clamp and provide isolation.

For anterior teeth no need to remove the rubber dam frame in order to take x-ray, just move it forward and place the film behind.

*after we finish RCT we place cotton and temporary filling , remove the rubber dam , then take  a radiograph using film holder. So the pre operative radiograph and the post operative one are taken using film holder.

Notice that you can’t dismiss your patient unless you have already developed the radiograph and show it to the supervisor .