#Prosthodontics 2 #3rd lecture #primary impression

A recap ….

Complete denture construction

- Primary impression

- Secondary impression

- Bite registration

- Try-in

- Insertion

- Post-insertion

In this lecture primary impression is going to be discussed

By definition primary impression is a negative form of the entire denture bearing area, In order to make a model for the purpose of diagnosis and fabrication of special tray.

To take or to make an impression?

It's to make an impression, because making an impression includes all of the important impression land marks.

Objectives of primary impression:

- To obtain or to make an impression for the whole denture bearing area of each arch

- To record full depth of the sulcus

- To obtain an impression in which all of the land marks are recorded

*please refer to impression landmarks of each jaw

*Instruments needed during the whole denture construction

- Edentulous impression stock trays (either perforated or not) , (plastic or metal)

- stanley knife (المشرط)

- water path

- impression material that we use for primary impression (compound or alginate or both)

Why do we use compound for primary impression (as a material of choice)?!

Compound classified as mucocompressive impression material, to compress the mucosa during primary impression

Some dentists use both alginate and compound (alginate wash) how?

It starts by taking the primary impression by compound then they use alginate as flowing material to register more details

- rubber bowel

- spatula (metal or plastic) for mixing

- apron

- gauze (4*4) for compound mixing and gauze (2*2) to dry the patients mouth during the selective grinding

- examination set

- wax knife

- flame

- patient file

*Ideal impression material for primary impression:

- not poison

- not irritant

- capable of producing an undercut

- capable for addition

- dimensionally stable

- set at room temp

- reasonable setting time

- should have pleasant taste

* The position of patient and the operator regarding taking the impression for the upper and the lower jaws

Lower Jaw: at the level of the shoulder, the operator should stand in the upright position, front right to the patient for right handed operator

Upper Jaw: at the level of the elbow, the operator should stand in the upright position, from behind the patient right to the patient for right handed operator

For left handed operators they should stand left to the patient

*Selection of the tray:

Requirements for stock tray:

- it should be rigid enough to carry the impression material

- it should cover the whole denture bearing area

- it should be easy to modify

Types of stock tray:

- plastic stock tray (solid or perforated)

- Metal (solid or perforated)

*procedure for selecting stock tray:

1- Trial and Error

2-use of the divider ((فرجار

Blunt ended, for the upper jaw from the distobuccul side of the arch, for the lower arch from the lingual side of the retomolar pad area

Then we transfer it to the trays, considering enough space for the material, in compound we need more space than the other materials because it’s a balky material

- Utilizing the existing denture , we but it inside the tray

Then we tray it inside the patients mouth

The lower tray: with index finger stretch one corner (half closing to increase muscle elasticity), the other end of the tray rotate it then seat it, support it by two hands by index on the premolar area , thumbs on the lower border of the mandible

The upper tray: with index finger stretch one corner (half closing to increase muscle elasticity), the other end of the tray rotate it then seat it, support it by index on the palate

*impression compound

Its thermoplastic rigid impression material softened when heated and hardened when cooled at oral temperature.

Composition:

- Resin

- Fillers (main bulk)

- Stearic acid

- Wax

Compound is also classified into

Low fusing: the one used in primary impression

High fusing: Green stick

Impression compound supplied by many forms usually the most common is the cake form (circular)

Color is usually brown

*precautions

What we need for primary impression: Rubber bowels, gauze 4*4, smear of Vaseline to prevent the compound from sticking to the hands

Ideal temperature for compound 60-65

Overheating = compound becomes very sticky, the volatile structure (stearic acid) will evaporate or leached out

Repetition of compound use more than 4 times (at relatively high temp) = compound become rough and granules tends to be clear, only the fillers stay in the compound

Under heating= may cause the granules to appear on the compound surface

*it can be disinfected by disinfectant solution

*Manipulation of the compound:

The compound should be softened in a hot water, after 3mins the compound should be removed and molden by two fingers to make a good homogenous form, color, texture

- It's preferable from clinical trials to take the lower impression before the upper for 2 reasons :

- To minimize any gag reflex (in gag reflex patients)

- To avoid an increase in the salivary secretion

*procedure:

1- Set the patient in a proper position, the operator should also sit in the proper position

2-the compound should be softened and shaped into a roll shape (for the lower impression) or ball shaped (for the lower jaw)

3-put it into the tray and then a shallow groove should be made to stimulate the center of the ridge

4-seat the tray in the patients mouth after check its temperature (not to burn the patient) with firm pressure on the premolar area for 30 seconds then carry on the functional movements, move the cheek and lips outwards

*always primary impression by compound gives an exaggerated border, so we have to carry on functional movements:

For the upper: upwards the down wards in then motional circular movement on the buccul side

*if the impression tends to be stuck, run your fingers around the periphery to break the seal

*once the impression is removed from the patients mouth put it under tap water ,to cool down, then dry it by air syringe then study your impression

*compound is capable for addition

1*insufficient depth in the posterior part maybe due to lack of compound in the tray, short flange, too little force or pressure on that area

2*insufficient depth on lingual, labial and buccul sulcus maybe due to lack of impression material, slight pressure, lack of flow of the material that it's not softened enough

Management of 1&2 you can add compound (softened piece of compound) on defective area (that’s can't be done in the case of alginate) then reseat the tray and mold it again

*impression tray should be symmetrical on the patients mouth (the frenum should bisect it)

*during the taking of the impression ask the patients to partially close his mouth to activate the coronoid process in the impression, on both primary and secondary impression (especially in secondary impression)

*impression must be disinfected and packaged in a plastic bag before pouring

THE END

*Please refer to the slides for more information

*forgive me for any mistake

Best wishes and love, Your colleague

Ahmad Tayel Khajiil